The goal of this part of the project is to use MATLAB to investigate the use of the Discrete Fourier Transform (DFT) for the spectral analysis of time series. This document contains the instructions needed to aid in your explorations. Several files will be needed in this project. They are provided on the project page. For those not familiar with MATLAB, there are some additional help files listed there including Dr. Herman's Wiki called MATLAB a brief introduction for students.  This assumes you have done the warm-up exercise already on plotting simple sinusoidal time series.

You can access MATLAB through Tealware, https://tealware.uncw.edu. MATLAB is under the Mathematics & Statistics folder, MATLAB2015a. Open the program. The below m-files can either be saved to the default directory on TIMMY, or the text can be copied and pasted into the editor and then saved with the original file name. Data and other files should be copied over to the same directory. You should see your files in the Current Folder panel on the left of the screen. [See the figure below.]

Running the m-files is done by typing the filename without the extension (.m) after the prompt (>>) in the Command Window, which is the middle panel of the MATLAB program.

You will keep track of your observations with your partner and submit the results on the last class day in report form. This project will count as your computer lab/project grade for the course. All work should be typed with double-spacing and 12 pt font. You will be expected to use correct English grammar and punctuation. This is a report and thus you will use proper sentence and paragraph formatting. You will be graded on the evidence of work, mathematical detail and understanding, proper exposition and neatness. Your work should also be supported with properly labeled and embedded plots and equations. Any references used should be cited as well. These reports will count towards the project component of your grade.

- Analysis of simple functions.

  - File ftex.m
This file is a MATLAB program implementing the discrete Fourier transform using trigonometric functions like that derived in the text. The input is a function, sometimes with different frequencies. The output is a plot of the data points and the function fit, the Fourier coefficients and the periodogram giving the power spectrum.

    - Save the file ftex.m in your working directory under MATLAB.

    - View the file by entering edit ftex Note how the first function is defined by the variable y.

    - Run the file by typing ftex in MATLAB's Command window.

    - Change the parameters in ftex.m, remembering the original ones. In particular, change the number of points (keeping them even), the frequency, and the record length. Note the effects. If you get an error, enter clear and try again. Always save the m-file after making any changes before running ftex.

    - Reset the parameters to the original values. What happens for frequencies of f0 = 5, 5.5 and 5.6? Is this what you expected?

    - Repeat the last set of frequencies for double the record length, T. Is there a change?

    - Reset the parameters. Put in frequencies of f0 = 20, 30, 40. What frequencies are present in the periodogram? Is this what you expected?

  - Look at sums of several trigonometric functions..

    - Reset the parameters.

    - Change the function in y to y=sin(2*pi*f0*t)+sin(2*pi*f1*t); and add a line defining f1. Start with f1 = 3; and look at several other values for the two frequencies. Try different amplitudes; for example,  3*sin(2*pi*f0*t) has an amplitude of 3. Record your observations.

    - Change one of the sines to a cosine. What is the effect? What happens when the sine and cosine terms have the same frequency?

  - Investigate non-sinusoidal functions.

    - Investigate the following functions:

      - y=t;

      - y=t.^2;

      - y=sin(2*pi*f0*(t-T/5))./(t-T/5); What is this function?

      - y(1,1:M)=ones(1,M); y(1,M+1:N)=zeros(1,N-M); Start with M=N/2; What is this function? How are the last two problems related? Do they relate to anything from earlier class lectures? What effect results from changing M?

      - Try multiplying the function in 4 by a simple sinusoid; for example, add the line y=sin(2*pi*f0*t).*y, for M=N/2. How does this affect what you had gotten for the sinusoid without multiplication?

- Use the FFT function.

In MATLAB there is a built in set of functions, fft and ifft for the computation of the Discrete Exponential Transform and its inverse using the Fast Fourier Transform (FFT). The files needed to do this are fanal.m or fanal2.m and fanalf.m.  Bring them into the editor and see what they look like. Note that fanal.m was split into the two files fanal2.m and fanalf.m. This will allow us to be confident when we later create new data files and then using fanalf.m.

- Test this set of functions for simple sine functions to see that you get results similar to Part 1. First run fanal and then fanal2. Is there any difference between these last two approaches?

- Analysis of data sets.

One often does not have a function to analyze. Some measurements are made over time of a certain quantity. This is called a time series. It could be a set of data describing things like the stock market fluctuations, sunspots activity, ocean wave heights, etc. Large sets of data can be read into y and small sets can be input as vectors. We will look at how this can be done. After the data is entered, one can analyze the time series to look for any periodic behavior, or its frequency content. In the two cases below, make sure you look at the original time series using plot(y,’+’).

  - Ocean Waves

In this example the data consists of monthly mean sea surface temperatures (oC) at one point over a 2 year period. The temperatures are placed in y as a row vector. Note how the data is continued to a second  line through the use of ellipsis. Also, one typically subtracts the average from the data. What affect should this have on the spectrum? Copy the following code into a new m-file called fdata.m and run fdata. Determine the dominant periods in the monthly mean sea surface temperature.

y=[7.6 7.4 8.2 9.2 10.2 11.5 12.4 13.4 13.7 11.8 10.1 ...

9.0 8.9 9.5 10.6 11.4 12.9 12.7 13.9 14.2 13.5 11.4 10.9 8.1];

n=length(y);

y=y-mean(y);

T=24;

dt=T/n;

fanalf(y,T);

  - Sunspots

Sunspot data was exported to a text file. Download the file sunspot.txt. You can open the data in Notepad and see the two column format. The times are given as years (like 1850). This example shows how a time series can be read and analyzed. From your spectrum, determine the major period of sunspot activity. Note that we first subtracted the average so as not to have a spike at zero frequency. Copy and paste into the editor and save as fanaltxt.m. Note: Copy and Paste of single quotes often does not work correctly. Retype the single quotes after copying.

[year,y]=textread('sunspot.txt','%d %f');

n=length(y);

t=year-year(1);

y=y-mean(y);

T=t(n);

dt=T/n;

fanalf(y',T);

- Analysis of sounds.

Sounds can be input into MATLAB. You can create your own sounds in MATLAB or sound editing programs like Audacity or Goldwave to create audio files. These files can be input into MATLAB for analysis.

Save the following sample code as fanalwav.m and save the first sound file.  This code shows how one can read in a WAV file. There will be two plots, the first showing the wave profile and the second giving the spectrum. Try some of the other wav files and report your findings. Note: Copy and Paste of single quotes often does not work correctly. Retype the single quotes after copying.

[y,NS,NBITS]=wavread('sound1.wav');

n=length(y);

T=n/NS

dt=T/n;

figure(1)

plot(dt*(1:n),y)

figure(2)

fanalf(y,T);

Several wav files are be provided for you to analyze. If you are able to hear the sounds, you can run them from MATLAB by typing sound(y,NS). In fact, you can even create your own sounds based upon simple functions and save them as wav files. For example, try the following code: Note: Copy and Paste of single quotes often does not work correctly. Retype the single quotes after copying.

smp=11025;

t=(1:2000)/smp;

y=0.75*sin(2*pi*440*t);

sound(y,smp,8);

wavwrite(y,smp,8, 'myfile.wav');

Try some other functions, using several frequencies. If you get a clipping error, then reduce the amplitudes you are using.