Homework #1

Chapter 1 Basic Concepts

Solution

Due: Monday, August 30th

Instructions: Complete the following problems showing all work.  Please write as legibly as possible.  If your handwriting is a problem use a text editor.

- What is the 8 bit binary (two’s complement) representation of each of the following signed decimal integers?

  - -72 => 1011 1000

    - abs -72 = 72,

    - convert to binary (repeated division)

| Division | Quotient | Remainder |
| --- | --- | --- |
| 72/2 | 36 | 0 (LSB) |
| 36/2 | 18 | 0 |
| 18/2 | 9 | 0 |
| 9/2 | 4 | 1 |
| 4/2 | 2 | 0 |
| 2/2 | 1 | 0 |
| 1 /2 | 0 | 1 |
|  |  | 0100 1000 |

    - apply 2’s complement because number was negative

      - toggle (reverse) bits 	=> 1011 0111

      - add 1			=> 1011 1000

  - -98 => 1001 1110

    - abs -98 = 98,

    - convert to binary (repeated division)

| Division | Quotient | Remainder |
| --- | --- | --- |
| 98/2 | 49 | 0 (LSB) |
| 49/2 | 24 | 1 |
| 24/2 | 12 | 0 |
| 12/2 | 6 | 0 |
| 6/2 | 3 | 0 |
| 3/2 | 1 | 1 |
| 1 /2 | 0 | 1 |
|  |  | 0110 0010 |

    - apply 2’s complement because number was negative

      - toggle (reverse) bits 	=> 1001 1101

      - add 1			=> 1001 1110

  - -5 => 1111 1011

    - abs -5 = 5,

    - convert to binary (repeated division)

| Division | Quotient | Remainder |
| --- | --- | --- |
| 5/2 | 2 | 1 (LSB) |
| 2/2 | 1 | 0 |
| 1 /2 | 0 | 1 |
|  |  | 0000 0101 |

    - apply 2’s complement because number was negative

      - toggle (reverse) bits 	=> 1111 1010

      - add 1			=> 1111 1011

- What is the 16-bit hexadecimal representation of each signed decimal integer?

  - -32 => FFE0

    - abs -32 = 32,

    - convert to hex (repeated division)

| Division | Quotient | Remainder |
| --- | --- | --- |
| 32/16 | 2 | 0 (LSB) |
| 2/16 | 0 | 2 |
|  |  | 0020 |

    - apply 2’s complement because number was negative

      - hex subtraction (F)	=> FFDF

      - add 1			=> FFE0

  - -60  => FFC4

    - abs -60 = 60,

    - convert to hex (repeated division)

| Division | Quotient | Remainder |
| --- | --- | --- |
| 60/16 | 3 | C (LSB) |
| 3/16 | 0 | 3 |
|  |  | 003C |

    - apply 2’s complement because number was negative

      - hex subtraction (F)	=> FFC3

      - add 1			=> FFC4

- Encode “Karl” in ASCII format in both hexadecimal and decimal.
4B 61 72 6C (hex) 
75 97 114 108 (dec)

- What is the decimal representation of the following signed binary numbers?

  - 10110101 => -75

    - Check MSB to determine if negative. Number is negative.

    - Negative, apply 2’s complement

      - toggle bits (1’s complement)	=> 0100 1010

      - add 1				=> 0100 1011

    - decode using expansion => 1x26 + 1x23 + 1x22 + 1 = - 75

  - 00101010 => 42

    - Check MSB to determine if negative. Number is non negative.

    - decode using expansion => 1x25 + 1x23 + 1x21  = 42

  - 11001100 => -52

    - Check MSB to determine if negative. Number is negative.

    - Negative, apply 2’s complement

      - toggle bits (1’s complement)	=> 0011 0011

      - add 1				=> 0011 0100

    - decode using expansion => 1x25 + 1x24 + 1x23 + 1 = - 52

- What is the binary representation of the following hexadecimal numbers?

  - E5B6AED7 => 1110 0101 1011 0110 1010 1110 1101 0111

  - 234B6D92 => 0010 0011 0100 1011 0110 1101 1001 0010

- What is the unsigned decimal representation of each hexadecimal integer?

  - 1C9 => 1x162 = 12x16 + 9 = 457

  - 6A5B => 6x163 + 10x162 + 5x16 + 11 = 27,227

- What is the minimum number of binary bits needed to represent each of the following unsigned decimal integers?

  - 65 => 7 bits  0 to 27- 1 (127)

  - 256 =>9 bits  0 to 29- 1 (511)

  - 127 7 bits  0 to 27- 1 (127)

- What is the hexadecimal representation of each of the following binary numbers

  - 1100 1101 0010 0001 => CD21

  - 0111 1001 0110 1010 => 796A

- What is the minimum number of binary bits needed to represent each of the following signed decimal integers? n bits has a range of -2n-1 to 2n-1 - 1

  - -64 => n = 7 -26 to 26-1 (-64 to 63)

  - -127 => n = 8 -27 to 27-1 (-128 to 127)

- Given the following binary sequence, mark the LSB and MSB

  - 0MSB 0 1LSB

  - 1 MSB 100 001 1 LSB

- What is the decimal representation of each of the following unsigned binary integers?

  - 1111 0011 => 243

  - 0110 1101 => 109

- What is the sum of each pair of binary integers?

  - 1010 1111 + 1101 1011 => 1 1000 1010

  - 1001 0111 + 0011 1100 => 1101 0011

- What is the sum of each pair of hexadecimal integers?

  - F8 1A + 02 E6 => FB00

  - 8B CD + 31 FA => BDC7

- How many bytes are in each of the following data types?

  - Word 		2 bytes

  - Doubleword	4 bytes

  - Quadword	8 bytes

- What is the value of the Boolean expression ¬F ˆ ¬T => T ^ F = F

- What is the value of the Boolean expression ¬(F ˇ T) => ¬F ˆ ¬T => T ^ F = F

- Create a truth table to show all possible inputs and outputs for the Boolean function described by (¬A ˆ ¬B)

| A | ¬A | B | ¬B | ¬A ˆ ¬B |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |